ASI 2027 ABSTRACT

We at Young Innovators have repurposed and recommissioned a 30-year-old television reflector dish antenna to conduct neutral hydrogen (H 21cm) studies in the RFI jungle of urban Bengaluru.

We have reused only the original aluminium frame of the 2.45m dish; everything else was custom-designed and built. We have built an Alt-Az mount for the dish, along with a VE4MA (popularly known as Kumar Feed) optimized for neutral hydrogen studies and the specific dish. The feed probe was made from 2.6mm solid copper wire about 5cm in length. The receiver combines a Nooelec Sawbird+, a bandpass filter, and an LNA (Low Noise Amplifier). This receiver setup is connected to the RTL-SDR via an LMR400 coax cable, which connects to a dedicated Ubuntu system running Virgo to capture data from the RTL-SDR.

To get the best results from the dish in an urban RFI, we ran multiple calibration and test runs on very bright sources such as Sagittarius and Cygnus over a few days to develop an algorithm/pipeline that provided sufficient SNR. During these test runs, we measured the dish's beamwidth using a solar drift scan to be ~5˚ and detected galactic neutral hydrogen with SNRs ranging from 20-25.

The 2.4m radio telescope is now being optimized to conduct a drift-scan hydrogen survey. We plan to make a sky map of neutral hydrogen and compare it with existing data. The project shows that even a small team with minimal resources can build a radio telescope from salvaged parts, giving amateur astronomers a path into research.