Radio-frequency interference (RFI) rejection is vital for urban radio astronomy, but standard threshold-based flagging risks removing genuine astronomical emission and biasing spectral-line measurements.

We examine this trade-off for Galactic neutral hydrogen (H1) observations using a refurbished 2.45 m television reflector in Bengaluru. Reusing only the original aluminum frame, the radio telescope features a custom-built, reinforced Alt-Azimuth mount, a custom-designed VE4MA (Kumar) feed, a Nooelec SAWbird+ H1 module, and an RTL-SDR receiver. During early testing, the telescope successfully detected Galactic H I with signal-to-noise ratios (SNRs) of 20-25 over a 60-minute integration.

While these initial detections are promising, extracting consistently clean spectra in a high-RFI environment requires aggressive data flagging. In this study,  we compare a conventional thresholding approach against a variant that estimates flagging limits only after modeling and subtracting candidate H I emission. Both methods are evaluated on identical recorded spectra with fixed baseline corrections.

To rigorously test performance, we inject synthetic H I profiles of varying amplitudes and widths into the raw data before flagging. We then assess both reduction methods based on recovery error and data contamination. By quantifying how accurately the injected signals are recovered versus how much interference remains, this characterization establishes the practical limitations of conventional RFI flagging.

Ultimately, this analysis maximizes signal preservation and directly informs the automated data reduction strategy for our drift-scan survey of the Milky Way's spiral arms.